" To select the ABEL device file for the GAL follow these rules

" 1. Registers are required.

"	Use the P16V8R.

" 2. No registers, but feedback from 8 I/O pins is required.

"	Use the P16V8R.

" 3. No registers, but feedback from 6 I/O pins is required.

"	Use the P16V8C.

" 4. No registers or feedback, but 8 terms per output are required.

"	Use the P16V8S.

The 16V8 has 64 bits available for user data, starting at fuse 2056.

Unfortunately the Data I/O does not have a decent way of reading it,

especially if the security bit is set.

fuses  "User Signature Word

[2056..2071] = 'V3';

[2072..2087] = '.0';

[2088..2103] = 'D-';

[2104..2119] = '87';

fuses  "User Signature Word

[2056..2087] = '052-';

[2088..2119] = '1139';

The 20V8 has 64 bits available for user data, starting at fuse 2568.

fuses  "User Signature Word

[2568..2583] = 'V3';

[2584..2599] = '.0';

[2600..2615] = 'D-';

[2616..2631] = '87';

fuses  "User Signature Word

[2568..2599] = '052-';

[2600..2631] = '1140';