TCPARTS.DOC		Jed Margolin  6/21/85

-----------------------------------------------

TomCat Parts Count

Totals:

Board #1 Main (Main Processor + AVG)  (66 + 79) = 145

Board #2 Aux (Sound + Math)	 	   =  48

or

Board #1 Main (Main Processor + Sound + Math)  (66 + 48) = 114

Board #2 AVG				  	    =  79

or

Board #1 Main Processor + AVG + Math (66+79+48) = 193

--------------------------------------------------------------------------------

Arrangement 1

Board 1 = Main Processor + AVG

3 Terminal  =  2

8 pin Dip   =  5

14 pin Dip  = 36

16 pin Dip  = 33

18 pin Dip  =  1

20 pin Dip  = 37

24 pin Wide = 12

28 pin Wide = 17

40 pin Wide =  1

64 pin Humungous = 1

Total Packages = 145

Board 2 = Sound Processor + Math Processor

8  pin Dip -  4

14 pin Dip -  5

16 pin Dip - 10

20 pin Dip - 17

24 pin wide - 3

28 pin wide - 4

40 pin wide - 5

---

48

--------------------------------------------------------------------------------

Arrangement 2

Board 1 = Main Board (Main Processor + Sound + Math)

8  pin Dip  =  4

14 pin Dip  = 21

16 pin Dip  = 15

20 pin Dip  = 40

24 pin Wide =  7

28 pin Wide = 21

40 pin wide =  5

64 pin Humungous =  1

-----

114

Board 2 =  AVG

3 Terminal =  2

8 pin Dip  =  5

14 pin Dip = 20

16 pin Dip = 28

18 pin Dip =  1

20 pin Dip = 14

24 pin Wide = 8

40 pin Wide = 1

----

79

--------------------------------------------------------------------------------

Arrangement 3

Board = Main Processor + AVG + Sound Processor + Math Processor

3 Terminal =  2

8  pin Dip  =  9

14 pin Dip  = 41

16 pin Dip  = 43

18 pin Dip =  1

20 pin Dip  = 54

24 pin Wide = 15

28 pin Wide = 21

40 pin wide =  6

64 pin Humungous =  1

--------

Total Packages = 193

Parts Count

Main Processor   #		Package			GND	+5V

-----------------------------------------------------------------------------

LS00     1		14 pin DIP		7	14

LS02	 1		14 pin DIP		7	14

AS04     1		14 pin DIP		7	14

LS04	 1		14 pin DIP		7	14

7406	 1		14 pin DIP		7	14

LS08	 1		14 pin DIP		7	14

LS14	 1		14 pin DIP		7	14

LS20	 1		14 pin DIP		7	14

LS32	 1		14 pin DIP		7	14

ALS74	 1		14 pin DIP		7	14

AS74	 1		14 pin DIP		7	14

LS74	 2		14 pin DIP		7	14

LS86	 1		14 pin DIP		7	14

LS138	 2		16 pin DIP		8	16

LS161	 2		16 pin DIP		8	16

LS244	13		20 pin DIP		10	20

LS245	 4		20 pin DIP		10	20

LS259	 1		16 pin DIP		8	16

LS299	 3		20 pin DIP		10	20

LS374	 3		20 pin DIP		10	20

LS393	 2		14 pin DIP		7	14

68010  	 1		64 pin Humungous	16,33	14,49

27256	16		28 pin Wide		14	28

137211

2K*8

RAM  	 4		24 pin Wide		12	24

ADC0809	 1		28 pin Wide		13	11

--------------------------------------------------------------------------------

TomCat Main Board - Processor

14 pin Dip  = 16

16 pin Dip  =  5

20 pin Dip  = 23

24 pin Wide =  4

28 pin Wide = 17

64 pin Humungous = 1

Total = 66

Main AVG     #		Package			GND	+5V

-------------------------------------------------------------------------------

LS00     1		14 pin DIP		7	14

S00	 1		14 pin DIP		7	14

LS02	 1		14 pin DIP		7	14

S02	 1		14 pin DIP		7	14

AS04     1		14 pin DIP		7	14

LS04	 1 		14 pin DIP		7	14

7407	 1		14 pin DIP		7	14

LS08	 1		14 pin DIP		7	14

S10	 1		14 pin DIP		7	14

LS11	 1		14 pin DIP		7	14

LS14	 1		14 pin DIP		7	14

LS20	 1		14 pin DIP		7	14

LS27	 1		14 pin DIP		7	14

LS32	 1		14 pin DIP		7	14

LS42	 1		16 pin DIP		8	16

LS74	 2		14 pin DIP		7	14

S74	 1		14 pin DIP		7	14

S86	 1		14 pin DIP		7	14

109	 1		16 pin DIP		8	16

LS139	 1		16 pin DIP		8	16

LS157	 2		16 pin DIP		8	16

LS161	 4		16 pin DIP		8	16

LS164	 1		14 pin DIP		7	14

LS174	 1		16 pin DIP		8	16

LS175	 6		16 pin DIP		8	16

LS191	 1		16 pin DIP		8	16

LS194	 6		16 pin DIP		8	16

LS244	 6		20 pin DIP		10	20

LS245	 4		20 pin DIP		10	20

S260	 1		14 pin DIP		7	14

LS273	 2		20 pin DIP		10	20

137211

2K*8

RAM  	 8		24 pin Wide		12	24

82HS137	 1		18 pin dip		9	18

7815	 1		3 terminal

7915 	 1		3 terminal

82S129	 1		16 pin Dip		8	16

VG Gate

Array	 1		40 Pin Wide

DAC-08	 2		16 pin dip

TL082	 5		8 pin Dip

AM6012	 2		20 pin DIP

LF13201	 2		16 pin DIP

---

79

-------------------------------------------------------------------------------

TomCat Main Board - AVG

3 Terminal =  2

8 pin Dip =   5

14 pin Dip = 20

16 pin Dip = 28

18 pin Dip =  1

20 pin Dip = 14

24 pin Wide = 8

40 pin Wide = 1

Total = 79

Aux Board

-------------

Sound Processor:

LS00 -3		14 pin Dip

LS04 		14 pin Dip

(1 1/2) LS74		14 pin Dip

(2) 	LS374		20 pin Dip

LS32 -3		14 pin Dip

(2) 	LS138		16 pin Dip

(1/2) 	LS139		16 pin Dip

LS161		16 pin Dip

(5)	LS191		16 pin Dip

(2) 	LS244		20 pin Dip

LS245		20 pin Dip

6502A		40 pin wide

6532A		40 pin wide

(2) 	2K*8 RAM	24 pin wide

(3) 	27256		28 pin wide

TMS-5220C	28 pin wide

(2) 	POKEY		40 pin wide

YM2151		24 pin wide

YM3012		16 pin Dip

(4) 	TL082		8 pin Dip

Math Processor:

TMS-320		40 pin wide

(4)	4k * 4 RAM	20 pin Dip

(4)	ALS244		20 pin Dip

(4)	ALS245		20 pin Dip

----------------------------------------------------

8  pin Dip -  4

14 pin Dip -  5

16 pin Dip - 10

20 pin Dip - 17

24 pin wide - 3

28 pin wide - 4

40 pin wide - 5

-----

Total	     48

Parts Count

Main Board - Main Processor

Sheet 1

Sheet 2

LS00 -1

AS04 -3

7406 -2

LS08 -1

LS14 -2

LS74

(2) LS161

(2) LS393

Sheet 3

68010

(2) LS244

(2) LS138

LS32

LS04 -2

Sheet 4

LS02 -1

AS04 -1

LS04 -3

LS08 -2

LS20

AS74

ALS74 -1

(2) LS138

LS259

Sheet 5

(8) 27256

LS244

Sheet 6

(8) 27256

LS244

Sheet 7

(4) 137211-001 2K*8 RAM

(2) LS245

Sheet 8

(3) LS244

LS374

ADC0809

Sheet 9

(3) LS299

LS86 -2

Sheet 10

(2 1/2) LS244

(2) LS245

Sheet 11

LS04 -2

LS74

(3 1/2) LS244

(2) LS374

------------------------------

Main Board - AVG

Sheet 12

(2) LS245

LS175

82HS137

Sheet 13

(6) LS244

(2) LS245

LS04 -1

AS04 -1

Sheet 14

(8) 137211-001 2K x 8 RAM

LS139

Sheet 15

7815  +15V Regulator

7915  -15V Regulator

Sheet 16

Sheet 17

LS02 -2

S02	 -1

LS04 -1

LS08 -1

S10

LS27 -1

LS32 -2

LS42

(1 1/2) LS74

S74

S86

109

(2) LS157

LS174

S260

82S129

Sheet 18

S00

LS08

LS14 -2

LS08

(2) LS175

(6) LS194

Gate Array

Sheet 19

LS00 -2

LS02 -2

S02 -2

S04 -1

LS08 -1

LS11 -1

LS20

LS32 -3

LS175

LS191

(4) LS161

Sheet 20

LS02 -2

LS04 -1

7407

S10 -1

LS164

(2) LS175

(2) LS273

(2) DAC-08

TL082 -1

Sheet 21

(2) AM6012

(2) LF13201

(4) TL082

LS04 -2

Main Board - Total		Package			GND	+5V

-----------------------------------------------------------------------------

LS00 -3  1		14 pin DIP		7	14

S00	 1		14 pin DIP		7	14

LS02	 2		14 pin DIP		7	14

S02	 1		14 pin DIP		7	14

AS04 -5  1		14 pin DIP		7	14

LS04	 2		14 pin DIP		7	14

S04	 1		14 pin DIP		7	14

7406	 1		14 pin DIP		7	14

7407	 1		14 pin DIP		7	14

LS08	 2		14 pin DIP		7	14

S10	 1		14 pin DIP		7	14

LS11	 1		14 pin DIP		7	14

LS14	 2		14 pin DIP		7	14

LS20	 2		14 pin DIP		7	14

LS27	 1		14 pin DIP		7	14

LS32	 3		14 pin DIP		7	14

LS42	 1		16 pin DIP		8	16

AS74	 1		14 pin DIP		7	14

ALS74	 1		14 pin DIP		7	14

LS74	 2		14 pin DIP		7	14

S74	 1		14 pin DIP		7	14

LS86	 1		14 pin DIP		7	14

S86	 1		14 pin DIP		7	14

109	 1		16 pin DIP		8	16

LS138	 4		16 pin DIP		8	16

LS139	 1		16 pin DIP		8	16

LS157	 2		16 pin DIP		8	16

LS161	 6		16 pin DIP		8	16

LS164	 1		14 pin DIP		7	14

LS174	 1		16 pin DIP		8	16

LS175	 6		16 pin DIP		8	16

LS191	 1		16 pin DIP		8	16

LS194	 6		16 pin DIP		8	16

LS244	19		20 pin DIP		10	20

LS245	 8		20 pin DIP		10	20

LS259	 1		16 pin DIP		8	16

S260	 1		14 pin DIP		7	14

LS273	 2		20 pin DIP		10	20

LS299	 3		20 pin DIP		10	20

LS374	 3		20 pin DIP		10	20

LS393	 2		14 pin DIP		7	14

ADC0809	 1		28 pin Wide		13	11

68010  	 1		64 pin Humungous	16,33	14,49

27256	16		28 pin Wide		14	28

137211

2K*8

RAM  	12		24 pin Wide		12	24

82S137	 1		18 pin Dip		9	18

82S129	 1		16 pin Dip		8	16

AM6012	 2		20 pin DIP

DAC-08	 2		16 pin DIP

LF13201	 2		16 pin DIP

TL082	 5		8 pin Dip

VG Gate

Array	 1		40 Pin Wide

7815	 1		3 terminal

7915 	 1		3 terminal

-------------------------------------------------------

Main Board (Processor + AVG)

3 Terminal  =  2

8 pin Dip   =  5

14 pin Dip  = 36

16 pin Dip  = 33

18 pin Dip  =  1

20 pin Dip  = 37

24 pin Wide = 12

28 pin Wide = 17

40 pin Wide =  1

64 pin Humungous = 1

Total = 145

Power Connections for TomCat Main Board

Ground   +5V

------   ----

LS00 -3  	7	14

S00	 	7	14

LS02	 	7	14

S02	 	7	14

AS04 -5  	7	14

LS04	 	7	14

S04	 	7	14

7406	 	7	14

7407	 	7	14

LS08	 	7	14

S10	 	7	14

LS11	 	7	14

LS14	 	7	14

LS20	 	7	14

LS27	 	7	14

LS32	 	7	14

LS42	 	8	16

AS74	 	7	14

ALS74	 	7	14

LS74	 	7	14

S74	 	7	14

LS86	 	7	14

S86	 	7	14

109	 	8	16

LS138	 	8	16

LS139	 	8	16

LS157	 	8	16

LS161	 	8	16

LS164	 	7	14

LS174	 	8	16

LS175	 	8	16

LS191	 	8	16

LS194	 	8	16

LS244		10	20

LS245	 	10	20

LS259	 	8	16

S260	 	7	14

LS273	 	10	20

LS299	 	10	20

LS374	 	10	20

LS393	 	7	14

ADC0809	 	13	11

68010  	 	16,33	14,49

27256		14	28

82S129	 	8	16

82S137	 	9	18

137211

2K*8

RAM  		12	24

Aux Board

-------------

Sound Processor:

LS00 -3		14 pin Dip

LS04 		14 pin Dip

(1 1/2) LS74		14 pin Dip

(2) 	LS374		20 pin Dip

LS32 -3		14 pin Dip

(2) 	LS138		16 pin Dip

(1/2) 	LS139		16 pin Dip

LS161		16 pin Dip

(5)	LS191		16 pin Dip

(2) 	LS244		20 pin Dip

LS245		20 pin Dip

6502A		40 pin wide

6532A		40 pin wide

(2) 	2K*8 RAM	24 pin wide

(3) 	27256		28 pin wide

TMS-5220C	28 pin wide

(2) 	POKEY		40 pin wide

YM2151		24 pin wide

YM3012		16 pin Dip

(4) 	TL082		8 pin Dip

Math Processor:

TMS-320		40 pin wide

(4)	4k * 4 RAM	20 pin Dip

(4)	ALS244		20 pin Dip

(4)	ALS245		20 pin Dip

----------------------------------------------------

TomCat Aux Board (Sound Processor + Math Processor)

8  pin Dip -  4

14 pin Dip -  5

16 pin Dip - 10

20 pin Dip - 17

24 pin wide - 3

28 pin wide - 4

40 pin wide - 5

-----

Total	     48

TomCat Parts Cost

Main Board	     512K Byte	256 K Byte

--------------------------------------------

68010		      	 20	   20

27256	$6.50 * 16 = 	104  	   52

137211  $3.   * 12 =	 36  	   36

AM6012  $6.   *  2 =  	 12  	   12

Gate Array	   =      6  	    6

other   $1.  * 114 = 	114  	  114

---- 	  ---

$292 	  240

Aux Board

---------

Sound Processor:

LS00 -3

LS04

(1 1/2) LS74

(2) 	LS374

LS32 -3

(2) 	LS138

(1/2) 	LS139

LS161

(5)	LS191

(2) 	LS244

LS245

$1*19		19

6502A					$3.00

6532A					$3.00

(2) 	2K*8 RAM		$3.00 * 2	$6.00

(3) 	27256			$6.50 * 3	$19.50

TMS-5220C				$6.50

(2) 	POKEY			$4.00 * 2	$8.00

YM2151					$15

YM3012					$5

(4) 	TL082					4

---------

$89

Math Processor:

TMS-320					$30.

(4)	4k * 4 RAMS, 70 ns	$3.00 * 4	$12

(4)	ALS244			$1.29 * 4	 5.16

(4)	ALS245			$1.69 * 4	 6.76

------

$53.92

AUX BOARD TOTAL APPROXIMATE:

$142.92

Main Board (512K) + Aux Board = 292 + 143 = $435