Applied Microsystems Emulator Setup Data:

SYSTEM:	  #1	USER= 0

#2	RESET CHAR= $01

#3	XON,XOFF = $11, $13

TERMINAL: #10	BAUD RATE= #14	(9600)

#11 	STOP BITS= 1

#12	PARITY= 1

#13   CRT LENGTH= #24

#14	TRANSPARENT MODE ESCAPE SEQUENCE= $1B,$1B  (esc,esc)

COMPUTER: #20  	BAUD RATE= #14 	(9600)

#21 	STOP BITS= 1

#22	PARITY= 1

#23	TRANSPARENT MODE ESCAPE SEQUENCE= $1B,$1B  (esc,esc)

#24	COMMAND TERMINATOR SEQUENCE= $0D,$00,$00

#25	UPLOAD RECORD LENGTH= #32

#26	DATA FORMAT= 5

#27	ACKNOWLEDGE CHAR= $06

CIT-101:  0101 0011 0000 1110 0000

1000 0001 0010 1000 0

0110 1010 001

Starting from scratch:

1. Put the parameter switch setting in Position 0. Disable parity on your

terminal so it will be able to talk to the Emulator.

2. Change parameters, except for #12 and #22 (parity).

3. When you do #12 it will say something about having to do a Reset. Do

'Control A'. At this point your terminal will stop talking to the Emulator

because they no longer agree about parity. Enable parity on your terminal

and you will be able to talk to the Emulator again.

4. Set #22 (Computer parity) and do 'Control A'.

5. Save everything by doing 'SAV'

6. Set the parameter switch setting to Position 1 (User 0 Terminal Control)

7. BE SURE TO SET OVS=1 . In this mode accesses to overlay memory will generate

their own /DTACK. There will also be no wait states.

If /DTACK is not working when you access memory:

On old systems you will get the error "NO TARGET CLOCK". This has

nothing to do with the processor clock!

On updated systems you will get either "NO TARGET CLOCK" or

something like "TARGET BUS ERROR" .

Therefore, if your target /DTACK is not working, you will not be able to

access the target memory space.

8. Assign the memory map as desired [ and save it by doing 'SAV (3)'].

Otherwise you might be surprised when you try to download a program.

And remember to initialize the Stack Pointer somewhere.

9. Set Fast Interrupt to on. [ON FST; SAV 4]

Jed Margolin