TMS-320 System for TomCat		Jed Margolin 	6/26/85

--------------------------

The TMS-320 System for TomCat consists of:

1. TMS-320

2. RAM (4K * 16)

3. Multiplexer (Buffers)

4. ALS244's hardwired to deliver BIOZ 0 instruction

5. A gate arrangement that produces a signal (/IDLE) when

the TMS-320 address is 0, 1, 2, or 3

To initialize:

The 68010:

sets /320RES to 0, (This also sets /BIO to 0)

sets /320RES to 1.

The TMS-320 will do a reset and go to address 0.

This will be detected and will substitute the ALS244 buffers

which are wired to deliver a BIOZ 0 instruction.

As long as /BIO is 0 the BIOZ 0 instruction will continuously

loop.

The 68010 can therefore access the RAM without bothering the TMS-320.

To run a program:

Load a "Branch Address" instruction at locations 04,05 in the RAM,

where "address" is the address of the program.

The 68010 strobes the 320BIO address which sets /BIO to '1'. When the

program counter gets to address 04, /IDLE will no longer be true.

(It is strobed by CLKOUT to make sure it is really valid.) When /IDLE*

goes high it will reset /BIO.

Each program ends with a Branch 0 instruction which causes the program

data to switch to the hardwired BIOZ 0 instruction.

/IDLE* tells the 68010 that the TMS-320 is done with the routine and

allows the 68010 to aceess the RAM without bothering the TMS-320.

To interrupt a program:

Forget it. I didn't put stuff in to do it.

You can do a Reset, but apparently the integrity of the data in

the internal data RAM isn't guaranteed after a Reset.

-----------------------------------------------------

RAM Timing using ALS Buffers:

CLKOUT to Address Valid = 50 nS

ALS244 Address Buffer   = 10 nS    -0.1 MA input (TMS-320 output is 2 MA)

(RAM)			=

ALS245 Data Buffer	= 10 nS    -0.1 MA input

Data Setup 		= 50 nS

--------

120 nS

RAM = TCYC - 121

12 MHZ: TCYC = 333 nS,  RAM = 213 nS

14 MHz: TCYC = 279 nS,  RAM = 159 nS

16 MHz: TCYC = 250 nS,  RAM = 130 nS

18 MHz: TCYC = 222 nS,  RAM = 102 nS

20 MHz: TCYC = 200 nS,	RAM = 80 nS

-----------------------------------------------------

RAM Timing using LS Buffers:

CLKOUT to Address Valid = 50 nS

LS244 Address Buffer    = 18 nS    -0.2 MA input (TMS-320 output is 2 MA)

(RAM)			=

LS245 Data Buffer	= 18 nS	   -0.2 MA input

Data Setup 		= 50 nS

--------

136 nS

RAM = TCYC - 136

12 MHZ: TCYC = 333 nS,  RAM = 197 nS

14 MHz: TCYC = 279 nS,  RAM = 143 nS

16 MHz: TCYC = 250 nS,  RAM = 114 nS

18 MHz: TCYC = 222 nS,  RAM =  86 nS

20 MHz: TCYC = 200 nS,	RAM =  64 nS

------------------------------------------------------------------

Therefore: With 70 nS RAM, Use ALS for 18 MHz and LS for 16 MHz.

With 55 nS RAM, LS can be used at 18 MHz.

------------------------------------------------------------------

For TMS320 system (11/15/85 prices):

8168-70   RAM	2.10 x 4 = 8.40  	(5K - Marshall)

74ALS244	1.33 x 6 = 7.98		(10K - Arrow)

74ALS245	1.33 x 4 = 5.32 	(10K - Arrow)

----

$21.70

8168-55   RAM	2.45 x 4 = 9.80  	(Inventory)

74LS244		.25  x 6 = 1.50		(Inventory)

74LS245		.28  x 4 = 1.12		(Inventory)

----

$12.42

The savings is $9.28 by using fast RAM but cheap buffers.

------------------------------------------------------------------

the end