Player Control with Both Force and Position Feedback.                   1 of 2

Jed Margolin	3/1/85

(Player)							(Motor)

|<-----L----->|

------------------                   --------------------

|    Shaft A       | OOOOOOOOOOOOOOO |     Shaft B        |

----|-------------     (spring)	 ----------------|---

|		    		 		 |

|<--Px-->|			        |<--Mx-->|

|	 |			        |	 |

|			        |

_________________|______________________________|_______________________

/ / / / / / / / / / / / / / / / / / / / / / / / / / / / / / / / / / / /

Distance Px is measured, for example, with a potentiometer.

Distance Mx is measured, for example, with a potentiometer.

Shaft 'A' is the end that interacts with the Player.

Shaft 'B' is connected to a high torque motor (for example, through a gear)

that controls displacement Mx.

Theory:

The length of the spring is proportional to (Mx+Px), therefore the Force

between shafts 'A' and 'B' is proportional to (Mx+Px) because of the spring.

By measuring Mx and Px we can calculate the Force.

With end 'A' free, the spring will assume its natural length and

(disregarding inertia) the position of Shaft 'A' will be determined by

the position of Shaft 'B' which is determined by the motor.

With end 'A' fixed, the force on it will be determined by the

length of the spring, which we can control by having the motor move

Shaft 'B'.

With end 'A' somewhere between free and fixed, we can control the

position within the limits of the maximum force we can use, and we can

control the force within the maximum limits of position.

Player Control with Both Force and Position Feedback.                   2 of 2

Jed Margolin	3/1/85

Applications:

1. Flight control for TomCat. The control loading would be controlled by

by the aerodynamic model. It would also offer a training mode where the

player would put his (her?) hands on the controls and would be taught

to fly. A speech synthesizer would be used to deliver the instructor's

commentary.

2. Flight control for the Flight Simulator.

3. Control for Road Runner.

When a joystick is used to control the velocity of an object, the force

on the stick resisting the player should start out high; then, as the

object increases its speed, the stick force should decrease. If the

object hits something, the stick should give the player a small jolt

in response. A Force/Position control would give the player the feeling

that he was really connected to the action on the screen.