TomCat Architectures in order of performance (and cost).

--------------------------------------------------------------------------------

A. Single Player Hardware:

Board 1

-------

68010 at 6 MHz

8K bytes RAM (some in Mostek Zeropower RAM)

256K Bytes ROM (max)

Vector Generator with 16K RAM, single hardware buffer

Board 2

-------

Math - TMS 320

Sound - 6502

2K RAM

48K ROM

(1) Pokey

(1) Yamaha (maybe 2)

--------------------------------------------------------------------------------

B. Single Player Hardware:

Board 1

-------

68010 at 8 MHz

8K bytes RAM (some in Mostek Zeropower RAM)

256K Bytes ROM (max)

Vector Generator with 32K RAM, double hardware buffer

Board 2

-------

Math - TMS 320

Sound - 6502

2K RAM

48K ROM

(1) Pokey

(1) Yamaha (maybe 2)

--------------------------------------------------------------------------------

C. Dual Player Hardware:

Board 1

-------

68010 at 8 MHz

8K bytes RAM (some in Mostek Zeropower RAM)

256K Bytes ROM (max)

Vector Generator with 32K RAM, double hardware buffer, switched output

to alternately drive two monitors.

Board 2

-------

Math - TMS 320

Sound - 6502

2K RAM

48K ROM

(2) Pokey (one for each player)

(2) Yamaha (one for each player)

--------------------------------------------------------------------------------

D. Dual Player Hardware:

Block 1

-------

68010 at 6 MHz

8K bytes RAM (some in Mostek Zeropower RAM)

256K Bytes ROM (max)

Block 2

-------

Vector Generator with 16K RAM, single hardware buffer (Player 1)

Block 3

-------

Vector Generator with 16K RAM, single hardware buffer (Player 2)

Block 4

-------

Math - TMS 320

Block 5

-------

Sound - 6502

2K RAM

48K ROM

(2) Pokey (one for each player)

(2) Yamaha (one for each player)

--------------------------------------------------------------------------------

D. Dual Player Hardware:

Block 1

-------

68010 at 8 MHz

8K bytes RAM (some in Mostek Zeropower RAM)

256K Bytes ROM (max)

Block 2

-------

Vector Generator with 32K RAM, double hardware buffer (Player 1)

Block 3

-------

Vector Generator with 32K RAM, double hardware buffer (Player 2)

Block 4

-------

Math - TMS 320

Block 5

-------

Sound - 6502

2K RAM

48K ROM

(2) Pokey (one for each player)

(2) Yamaha (one for each player)

--------------------------------------------------------------------------------

E. Dual Player Hardware:  [2 single player hardware sets]

Board 1 (Player 1)

------------------

68010 at 6 MHz

8K bytes RAM (some in Mostek Zeropower RAM)

256K Bytes ROM (max)

Vector Generator with 16K RAM, single hardware buffer

Parallel Game Interface

Board 2 (Player 1)

------------------

Math - TMS 320

Sound - 6502

2K RAM

48K ROM

(1) Pokey

(1) Yamaha (maybe 2)

Board 3 (Player 2)

------------------

68010 at 6 MHz

8K bytes RAM (some in Mostek Zeropower RAM)

256K Bytes ROM (max)

Vector Generator with 16K RAM, single hardware buffer

Parallel Game Interface

Board 4 (Player 2)

------------------

Math - TMS 320

Sound - 6502

2K RAM

48K ROM

(1) Pokey

(1) Yamaha (maybe 2)

--------------------------------------------------------------------------------

F. Dual Player Hardware:

Board 1 (Player 1)

------------------

68010 at 8 MHz

8K bytes RAM (some in Mostek Zeropower RAM)

256K Bytes ROM (max)

Vector Generator with 32K RAM, double hardware buffer

Parallel Game Interface

Board 2 (Player 1)

------------------

Math - TMS 320

Sound - 6502

2K RAM

48K ROM

(1) Pokey

(1) Yamaha (maybe 2)

Board 3 (Player 2)

------------------

68010 at 8 MHz

8K bytes RAM (some in Mostek Zeropower RAM)

256K Bytes ROM (max)

Vector Generator with 32K RAM, double hardware buffer

Parallel Game Interface

Board 4 (Player 2)

------------------

Math - TMS 320

Sound - 6502

2K RAM

48K ROM

(1) Pokey

(1) Yamaha (maybe 2)

68010 prices 3/11/85:		10 MHz (Plastic)      12.5 MHz (Ceramic)

----------------      ------------------

5K		   $29.50		  $70.50

20K		   $25.

We are currently using 68010-L8	in quantities of 20K for $20.

What I finally decided on:	One Multilayer Board

Main Processor:

68010  	@6MHz

512K	EPROM  (16 x 27256)

8K	RAM    (includes 4K in MOSTEK ZeroPower RAM)

A/D Converter

8 bit option switch

Switch Inputs (7)

Hardware Pseudo-Random Number Generator (23 bit Linear Feedback

Shift Register)

16 Bit Parallel Game-Link Interface

Analog Vector Generator:

Analog Vector Generator with 16K RAM

Sound Processor:

6502 processor @1.79 MHz

8K*8 RAM

32K Program and Music EPROM  (1 x 27256)

64K Speech EPROM (2 x 27256)  [Program loads an address into a counter,

Every SROM READ increments the counter]

Yamaha Chip Set

Pokey	(2)

TMS-5220C Speech Synthesizer

Coin Switch Inputs

Coin Counter Outputs

8 Bit Option Switch

8 Bit Parallel Interface to 68010

Math Processor:

TMS-320 Digital Signal Processing IC (For faster math calculations)

4K Words RAM for TMS-320 (Interfaced to 68010)

---------

end