TomCat	 Jed Margolin	1/14/86

------------------------------------

Hardware Specifications:

Main Processor:

68010  	@6MHz

512K	EPROM  (16 x 27256)

8K	RAM    (4 x 2K*8)

A/D Converter

8 bit option switch

Switch Inputs (7)

Hardware Pseudo-Random Number Generator (23 bit Linear Feedback

Shift Register)

17 Bit Parallel Game-Link Interface

Analog Vector Generator:

Star Wars Analog Vector Generator with 16K RAM

Sound Processor:

6502 processor @1.79 MHz

8K*8 RAM

32K Program and Music EPROM  (1 x 27256)

64K Speech EPROM (2 x 27256)  [Program loads an address into a counter,

Every SROM READ increments the counter]

Yamaha Chip Set

Pokey	(2)

TMS-5220C Speech Synthesizer

Coin Switch Inputs

Coin Counter Outputs

8 Bit Option Switch

8 Bit Parallel Interface to 68010

Math Processor:

TMS-320 Digital Signal Processing IC (For faster math calculations)

4K Words RAM for TMS-320 (Interfaced to 68010)

BOARD #1: Current = 4.1 Amps (NO EPROMs)

PCB Cost Estimate: $350 - $400 stuffed and tested (512K Bytes Program Memory)

Software (as of 4/29/86)

--------

Runway:

Main Algorithm

Strobe Lights

VASI Lights

Terrain Objects

Independent control of enemy ship

Firing the Guns

Macros for Object Drawing Routines

Macros for VG Alphanumerics

Objects:

Cube

Rocket

Airplane

BattleZone: Tank, Flying Saucer, Buzz Bomber

ABZ: Helicopter

Horizon

Headup Up Display:

Heading, Airspeed, Altitude, Thrust

Grid (made possible by full 3D Clipping)

Stars

Crude Aerodynamics

Control: Regular

Computer Assisted Controls

Two Game, Two Player Game Link

VG and Monitor Self-Test Patterns, menu selected (from table)

Player Options, menu selected (from table)

Hardware:

Analysis of 68010 timing.

Generation of real timing strobes for the hardware.

Vector Generator Memory Contention Circuit.

--------------------------------------------------------------------------------

Possible Configurations:

1. Kit for Star Wars (Hope not)

2. Full Game - One Player (with Link option)

3. Full Game - Two Players (two monitors, two power supplies, two game

boards)

Hardware Next:

1. 	Gary has list of PCB corrections and changes.

2.	AMI scrapped all the tooling and documentation on the Vector Generator

Gate Array.

Choices:

1. Do oops! board

2. Do new Gate Array

3. Do Full Custom

3.	Need Color XY Monitor.

*===============================================================================

*===============================================================================

XDEF	K1,FTEMP1

K1	DC.W	$4000

FTEMP1:	RTS

MOVEA.W	#UV2,A0

MOVE.W	AX(A0),D0

MULS	K1,D0

ASL.L	#2,D0

SWAP	D0		* XP	4000 MAX

MOVE.W	BX(A0),D1

MULS	K1,D1		* YP	1000 0000 MAX

DIVS	D0,D1		* YP/XP 4000 MAX

ASR.W	#5,D1		*	200 MAX	   DX

MOVE.W	CX(A0),D2

MULS	K1,D2		* ZP	1000 0000 MAX

DIVS	D0,D2		* ZP/XP	4000 MAX

ASR.W	#5,D2		*	200 MAX	   DY

MOVE.L	VG_PTR,A0

AND.W	#$1FFF,D2

MOVE.W	D2,(A0)+

AND.W	#$1FFF,D1

OR.W	#$3000,D1

MOVE.W	D1,(A0)+

MOVE.W	#$8040,(A0)+

MOVE.L	A0,VG_PTR

RTS

*-------------------------------------------------------------------------------

As of 9/4/85 (Wirewrap Hardware with 16 MHz TMS-320):

WITH TMS320		68010 ONLY

-----------		----------

TC68		41C	1052		3D6	982

TESTS		B42	2882		64C	1612

MATH		740	1856		8AE	2222

MATH2		5D8	1496	---	5D8	1496

CLIP		27C	636		774	1908

AERO		59C	1436	---	59C	1436

DRAW		E56	3670	---	E56	3670

VGIMAGE		3D8	 984	---	3D8	 984

HEADUP		4A2	1186	---	4A2	1186

PICS		69E	1694	---	69E	1694

-----			-----

16892			17190

================================================================================