.TITLE TCSTATE

.IF NE,0

TomCat State PROM for AVG memory arbitration.

Inputs:

1.5 MHz   3 MHz	=   T_STATE

-------   ------    -------

0	    0	      T0

0	    1	      T1

1	    0	      T2

1	    1	      T3

S4	  /VMEM	 =   VGSEL

---     ------     -----

0	    0	       0

0	    1	       0

1	    0	       1

1	    1	       0

ST2	    Q3   =   VFETCH

---       --       ------

0        0	       0

0        1          1

1        0          0

1        1          0

A9		1.5 MHz    }    T0, T1, T2, T3		Time State

A8		3 MHz      }

A7		S4	   }    VGSEL			1 = 68010 wants VG memory

A6		/VMEM	   }

A5		ST2        }    VFETCH			1 = VG wants VG memory

A4		Q3         }

A3  (Q3)	VG-STALL				1 = Stall VG for current cycle

A2  (Q2)    	VG-DTACK				1 = DTACK for 68010

A1  (Q1)    	VBUFF					1 = Enable Address and Data Buffers for 68010 access.

A0  (Q0)    	V68					1 = 68010 VG Access Request

VFETCH is not guaranteed to be valid at T0

__    __    __    __    __    __    __    __    __    __    __    __

12 MHz	   \__/  \__/  \__/  \__/  \__/  \__/  \__/  \__/  \__/  \__/  \__/  \_

_____       _____       _____       _____       _____       _____

6 MHz	__/     \_____/     \_____/     \_____/     \_____/     \_____/     \__

T0          T1          T2          T3          T0

^		  ^	      ^		  ^           ^

________             ___________             ___________             __

3 MHz	        \___________/           \___________/           \___________/

________                         _______________________

1.5 MHz	        \_______________________/                       \______________

T0          T1          T2          T3          T0

^		  ^	      ^		  ^           ^

_______________________   Latches, Strobes      ________

LCLK,ST3 _____________/		              \_______________________/

.ENDC

;If the 68010 wants the memory and the VG is not using it, give it to the 68010.

;

;    VGSEL VFETCH VGSTALL VGDTACK  VBUFF    V68   >>  VGSTALL VGDTACK  VBUFF   V68

;    ----- ------ ------- -------  -----   -----      ------- -------  -----   -----

T0     1     x	     x       x       x       x           0       1       1       1

T1     0     0	     0       1       1       1           0       0       1       1

T2     0     0	     0       0       1       1	         0       0       0       0

T3     0     0       0       0       0       0           0       0       0       0

;----------------------------------------------------------------------------------------

T1     1     0	     x       x       x       x           0       1       1       1

T2     0     0	     0       1       1       1           0       0       1       1

T3     0     0	     0       0       1       1	         0       0       0       0

T0     0     x       0       0       0       0           0       0       0       0

;----------------------------------------------------------------------------------------

T2     1     0	     x       x       x       x           0       1       1       1

T3     0     0	     0       1       1       1           0       0       1       1

T0     0     x	     0       0       1       1	         0       0       0       0

T1     0     0       0       0       0       0           0       0       0       0

;----------------------------------------------------------------------------------------

T3     1     0	     x       x       x       x           0       1       1       1

T0     0     x	     0       1       1       1           0       0       1       1

T1     0     0	     0       0       1       1	         0       0       0       0

T2     0     0       0       0       0       0           0       0       0       0

;----------------------------------------------------------------------------------------

;----------------------------------------------------------------------------------------

;If the VG wants the memory and the 68010 is not using it, give it to the VG.

;    VGSEL VFETCH VGSTALL VGDTACK  VBUFF    V68   >>  VGSTALL VGDTACK  VBUFF    V68

;    ----- ------ ------- -------  -----   -----      ------- -------  -----   -----

T0     0     x	     0       0       0       0           0       0       0       0

T1     0     1	     0       0       0       0           0       0       0       0

T2     0     1	     0       0       0       0	         0       0       0       0

T3     0     1       0       0       0       0           0       0       0       0

;----------------------------------------------------------------------------------------

;----------------------------------------------------------------------------------------

;If the 68010 wants the memory during 1.5 MHz low, and the VG wants it, give it to the 68010 and stall the VG.

;    VGSEL VFETCH VGSTALL VGDTACK  VBUFF    V68   >>  VGSTALL VGDTACK  VBUFF    V68

;    ----- ------ ------- -------  -----   -----      ------- -------  -----   -----

T0     1     x	     0       0       0       0           0       1       1       1

T1     0     1	     0       1       1       1           1       0       1       1

T2     0     1	     1       0       1       1	         1       0       0       0

T3     0     1       1       0       0       0           1       0       0       0

T0     0     x       1       0       0       0	         0       0       0       0

T1     0     1       0       0       0       0           0       0       0       0

;----------------------------------------------------------------------------------------

T1     1     1	     0       0       0       0           1       1       1       1

T2     0     1	     1       1       1       1           1       0       1       1

T3     0     1	     1       0       1       1	         1       0       0       0

T0     0     x       1       0       0       0           0       0       0       0

T1     0     1       0       0       0       0           0       0       0       0

;----------------------------------------------------------------------------------------

;If the 68010 wants the memory during 1.5 MHz high, and the VG is using it, make the 68010 wait until T0.

;Next VG cycle is memory.  Stall the next VG cycle (even if it isn't strictly necessary).

;    VGSEL VFETCH VGSTALL VGDTACK  VBUFF    V68   >>  VGSTALL VGDTACK  VBUFF    V68

;    ----- ------ ------- -------  -----   -----      ------- -------  -----   -----

T2     1     1	     x       x       x       x           0       0       0       1

T3     0     1	     0       0       0       1           0       0       0       1

T0     0     x	     0       0       0       1	         0       1       1       1

T1     0     1       0       1       1       1           1       0       1       1

T2     0     1	     1       0       1       1	         1       0       0       0

T3     0     1       1       0       0       0           1       0       0       0

T0     0     x       1       0       0       0           0       0       0       0

T1     0     1       0       0       0       0           0       0       0       0

;----------------------------------------------------------------------------------------

T3     1     1	     x       x       x       x           0       0       0       1

T0     0     x	     0       0       0       1           0       1       1       1

T1     0     1	     0       1       1       1	         1       0       1       1

T2     0     1       1       0       1       1           1       0       0       0

T3     0     1       1       0       0       0           1       0       0       0

T0     0     x       1       0       0       0           0       0       0       0

T1     0     1       0       0       0       0           0       0       0       0

;----------------------------------------------------------------------------------------

;----------------------------------------------------------------------------------------

;If the 68010 wants the memory during 1.5 MHz high, and the VG is using it, make the 68010 wait until T0.

;Next VG cycle is not memory.

;    VGSEL VFETCH VGSTALL VGDTACK  VBUFF    V68   >>  VGSTALL VGDTACK  VBUFF    V68

;    ----- ------ ------- -------  -----   -----      ------- -------  -----   -----

T2     1     1	     x       x       x       x           0       0       0       1

T3     0     1	     0       0       0       1           0       0       0       1

T0     0     x	     0       0       0       1	         0       1       1       1

T1     0     0       0       1       1       1           0       0       1       1

T2     0     0	     0       0       1       1	         0       0       0       0

;----------------------------------------------------------------------------------------

T3     1     1	     x       x       x       x           0       0       0       1

T0     0     x	     0       0       0       1           0       1       1       1

T1     0     0	     0       1       1       1	         0       0       1       1

T2     0     0       0       0       1       1           0       0       0       0

;----------------------------------------------------------------------------------------

;----------------------------------------------------------------------------------------

;Everything else 					 0       0       0       0