VG Memory Contention:

6502/6809:

The 6502/6809 are synchronous processors, as is the State Machine.

A memory access is always initiated at the same point of the 1.5 MHz clock. If

both the processor and the State Machine start a VG memory access, the

processor has priority and State Machine is stalled for that cycle.

When the State Machine is doing a memory access, ST2 is low. If ST2 is

low during /VMEM, the State Machine is stalled. However, ST2 is also low

during CENTER and VECTOR execution. Stalling the State Machine during this time

does not affect the execution of the Vector or Center instructions; the State

Machine would otherwise be spinning.

68010:

This is an asynchronous processor; the memory access can begin during

any 6 MHz clock.

If the State Machine is not accessing memory, the 68010 can have

the bus with no wait states.

If the 68010 is not accessing memory, the State Machine can have the

bus with no stall cycles.

If the State Machine is in the middle of a memory access (LCLK Low),

the 68010 will have to wait if it wants memory.

If the 68010 is accessing memory at the beginning of a State Machine

memory cycle, the State Machine will have to stall for that cycle.

Because ST2 is low during CENTER and VECTOR executions as well as

memory accesses, it would make the 68010 wait unnecessarily. If it made it

wait two extra 6MHz periods (333 ns) and the vector memory was divided into

two 8K byte buffers (4K words), it would waste 1.3 ms of 68010 time. Even

0.6 ms (4K bytes=2K words) is too much.

When the State Machine is stalled by a VECTOR or CENTER execution,

all of the Q states are low. (Q3 low prevents ST3 from producing Latch or

Strobe pulses.) The State Machine is doing a memory access only during

ST2 low and Q3 high. Both are stable 94 ns into LCLK high.

TomCat AVG Timing (13.9 ns/div)

@@@@  @@@@  @@@@  @@@@  @@@@  @@@@  @@@@  @@@@  @@@@  @@@@  @@@@  @@@

12 MHz     @  @  @  @  @  @  @  @  @  @  @  @  @  @  @  @  @  @  @  @  @  @  @

@@  @@@@  @@@@  @@@@  @@@@  @@@@  @@@@  @@@@  @@@@  @@@@  @@@@  @@@@

@@    @@@@@@@     @@@@@@@     @@@@@@@     @@@@@@@     @@@@@@@     @@@@@@@

6 MHz	 @    @     @     @     @     @     @     @     @     @     @     @

(LS161) @@@@@@     @@@@@@@     @@@@@@@     @@@@@@@     @@@@@@@     @@@@@@@

@@	   @@@@@@@@@@@@@           @@@@@@@@@@@@@           @@@@@@@@@@@@@

3 MHz	@	   @  	       @	   @	       @	   @

@@@@@@@@@@@@           @@@@@@@@@@@@@           @@@@@@@@@@@@@

@@	               @@@@@@@@@@@@@@@@@@@@@@@@@

1.5 MHz	@  		       @		       @

@@@@@@@@@@@@@@@@@@@@@@@@                       @@@@@@@@@@@@@@@@@@@@@@@@

@@@@@@@@@@@@@@@@@@@@@@@@@   		      @@@@@@@@@@@@@@@

LCLK, ST3     @			      @	 Latches, Strobes     @

(LS74)	@@@@@@@			      @@@@@@@@@@@@@@@@@@@@@@@@@

LS174=30, LS157=14, PROM=50, Total=94ns

| 94  |   Q valid              		      |

ST3 low to high increments GA address counter

| 168 ns 	  | GA address valid		       |

Stalls high

@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@

@			      @	 		      @

LCLK, ST3     @			      @	 Latches, Strobes     @

(LS74)	@@@@@@@	  stall before >|     @@@@@@@@@@@@@@@@@@@@@@@@@

------------------------------------------------------------------------------

__    __    __    __    __    __    __    __    __    __    __    __

12 MHz	   \__/  \__/  \__/  \__/  \__/  \__/  \__/  \__/  \__/  \__/  \__/  \_

_____       _____       _____       _____       _____       _____

6 MHz	__/     \_____/     \_____/     \_____/     \_____/     \_____/     \__

________             ___________             ___________             __

3 MHz	        \___________/           \___________/           \___________/

________                         _______________________

1.5 MHz	        \_______________________/                       \______________

_______________________   Latches, Strobes      ________

LCLK,ST3 _____________/		              \_______________________/

stall before >|

| 94  |   Q valid              		      |

| 168 ns 	  | GA address valid		      |

ST2 low and Q3 high. Both are stable 94 ns into LCLK high.

Timing Relationship between 68010 and Vector Generator

__    __    __    __    __    __    __    __    __    __    __    __

12 MHz	   \__/  \__/  \__/  \__/  \__/  \__/  \__/  \__/  \__/  \__/  \__/  \_

S0    S1    S2    S3    S4    S5    S6    S7    S0

_____       _____       _____       _____       _____       _____

6 MHz	__/     \_____/     \_____/     \_____/     \_____/     \_____/     \__

:

:___________

RVA 	__________________________/           \_________________

________             ___________             ___________             __

3 MHz	        \___________/     :     \___________/           \___________/

________                         _______________________

1.5 MHz	        \_______________________/                       \______________

:

_______________________   Latches, Strobes      ________

LCLK,ST3 _____________/		              \_______________________/

stall before >|

| 94  |   Q valid              		      |

| 168 ns 	  | GA address valid		      |

__    __    __    __    __    __    __    __    __    __    __    __

12 MHz	   \__/  \__/  \__/  \__/  \__/  \__/  \__/  \__/  \__/  \__/  \__/  \_

S0    S1    S2    S3    S4    S5    S6    S7    S0

_____       _____       _____       _____       _____       _____

6 MHz	__/     \_____/     \_____/     \_____/     \_____/     \_____/     \__

:

___________

RVA 	______________________________________/           \___________________

________             ___________      :      ___________             __

3 MHz	        \___________/           \___________/           \___________/

________                         _______________________

1.5 MHz	        \_______________________/                       \______________

_______________________:  Latches, Strobes      ________

LCLK,ST3 _____________/		              \_______________________/

stall before >|

| 94  |   Q valid              		      |

| 168 ns 	  | GA address valid		      |

__    __    __    __    __    __    __    __    __    __    __    __

12 MHz	   \__/  \__/  \__/  \__/  \__/  \__/  \__/  \__/  \__/  \__/  \__/  \_

S0    S1    S2    S3    S4    S5    S6    S7    S0

_____       _____       _____       _____       _____       _____       _____       _____       _____       _____

6 MHz	__/     \_____/     \_____/     \_____/     \_____/     \_____/     \_____/     \_____/     \_____/     \_____/     \_

:

___________

RVA 	__________________________________________________/           \___________

________             ___________             ___________             ___________             ___________

3 MHz	        \___________/           \___________/     :     \___________/           \___________/           \______________

________                         _______________________                         _______________________

1.5 MHz	        \_______________________/                 :     \_______________________/                       \______________

_______________________   Latches, Strobes      _______________________   Latches, Strobes      ________

LCLK,ST3 _____________/		              \_______________________/		              \_______________________/

:

stall before >|                                 stall before >|

| 94  |   Q valid              		      | 94  |   Q valid              		      |

| 168 ns 	  | GA address valid		      | 168 ns 	  | GA address valid		      |

__    __    __    __    __    __    __    __    __    __    __    __    __    __    __    __    __    __

12 MHz	   \__/  \__/  \__/  \__/  \__/  \__/  \__/  \__/  \__/  \__/  \__/  \__/  \__/  \__/  \__/  \__/  \__/  \_

S0    S1    S2    S3    S4    S5    S6    S7    S0

_____       _____       _____       _____       _____       _____       _____       _____

6 MHz	__/     \_____/     \_____/     \_____/     \_____/     \_____/     \_____/     \_____/     \__

:

___________

RVA 	_____________________________________________________________/           \__________________

________             ___________             ___________      :      ___________             ___________

3 MHz	        \___________/           \___________/           \___________/           \___________/           \___________/

________                         _______________________      :                  _______________________

1.5 MHz	        \_______________________/                       \_______________________/                       \______________

_______________________   Latches, Strobes      _______________________   Latches, Strobes      ______

LCLK,ST3 _____________/		              \_______________________/ 	              \_______________________/

:

stall before >|                                 stall before >|

| 94  |   Q valid              		      | 94  |   Q valid              		      |

| 168 ns 	  | GA address valid		      | 168 ns 	  | GA address valid		      |

68010 Timing

No Wait:

S0        S1        S2        S3        S4        S5        S6        S7        S0

_________           _________           _________           _________           _________

CLK	_/         \_________/         \_________/         \_________/         \_________/         \_________/

:

________________________________                   :

/AS	                     \__________\_____________________________________________________________________

___________________

S4	______________________/________/          \____________________________

_________________________________________          :           ___________________

/RVA	                                         \____________________/

_________________________________________          :           ___________________

/DTACK	                                         \____________________/

:

____________________________________________________                   :  ___________________

/RWS	                                                    \____________________/

:

__________________________________________

\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\_________________________________________///////////

:  70  :Data Out Valid (W)        Latch Data In (R)

^

One Wait:

S0        S1        S2        S3        S4        S5        S6        S7        S8       S9        S0

_________           _________           _________           _________           _________           _________

CLK	_/         \_________/         \_________/         \_________/         \_________/         \_________/         \_

:

_____________________________________________________________          :           _______________________________________

/DTACK	                                                             \____________________/

________________________________________________________________________                   :  ___________________

/RWS	                                                                        \____________________/

________________________________

/AS	                     \__________\_____________________________________________________________________

:

______________________________________________________________

\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\_______________________________________________________________

:  70  :Data Out Valid (W)                           Latch Data In (R)

^

Two Wait:

S0        S1        S2        S3        S4        S5        S6        S7        S8       S9        S10       S11

_________           _________           _________           _________           _________           _________           _

CLK	_/         \_________/         \_________/         \_________/         \_________/         \_________/         \_________/

:

_________________________________________________________________________________          :           ____________________

/DTACK	                                                                                 \____________________/

___________________________________________________________________________________________                   :  _________

/RWS	                                                                                           \____________________/

________________________________                                                                              :

/AS	                     \__________\________________________________________________________________________________________

:

_________________________________________________________________________________

\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\\_________________________________________________________________________________

:  70  :Data Out Valid (W)                                              Latch Data In (R)

^

Inputs:

1.5 MHz   3 MHz	=   T_STATE

-------   ------    -------

0	    0	      T0

0	    1	      T1

1	    0	      T2

1	    1	      T3

S4	  /VMEM	 =   VGSEL

---     ------     -----

0	    0	       0

0	    1	       0

1	    0	       1

1	    1	       0

ST2	    Q3   =   VFETCH

---       --       ------

0        0	       0

0        1          1

1        0          0

1        1          0

A9		1.5 MHz    }    T0, T1, T2, T3		Time State

A8		3 MHz      }

A7		S4	   }    VGSEL			1 = 68010 wants VG memory

A6		/VMEM	   }

A5		ST2        }    VFETCH			1 = VG wants VG memory

A4		Q3         }

A3  (Q3)	VG-STALL				1 = Stall VG for current cycle

A2  (Q2)    	VG-DTACK				1 = DTACK for 68010

A1  (Q1)    	VBUFF					1 = Enable Address and Data Buffers for 68010 access.

A0  (Q0)    	V68					1 = 68010 VG Access Request

VFETCH is not guaranteed to be valid at T0

__    __    __    __    __    __    __    __    __    __    __    __

12 MHz	   \__/  \__/  \__/  \__/  \__/  \__/  \__/  \__/  \__/  \__/  \__/  \_

_____       _____       _____       _____       _____       _____

6 MHz	__/     \_____/     \_____/     \_____/     \_____/     \_____/     \__

T0          T1          T2          T3          T0

^		  ^	      ^		  ^           ^

________             ___________             ___________             __

3 MHz	        \___________/           \___________/           \___________/

________      :           :      _______________________

1.5 MHz	        \_______________________/                       \______________

T0          T1          T2          T3          T0

^		  ^	      ^		  ^           ^

_______________________   Latches, Strobes      ________

LCLK,ST3 _____________/		              \_______________________/

If the 68010 wants the memory and the VG is not using it, give it to the 68010.

VGSEL VFETCH VGSTALL VGDTACK  VBUFF    V68   >>  VGSTALL VGDTACK  VBUFF    V68    	   T Addr  Data   (State)

----- ------ ------- -------  -----   -----      ------- -------  -----   -----       ------  ----   -------

T0:    1     x	     x       x       x       x           0       1       1       1	   0 2X	    7	   1 (1,26)

:											   0 3X     7      2 (2,27)

T1:    0     0	     0       1       1       1           0       0       1       1	   1 07     3	   3 (3,63,68)

T2:    0     0	     0       0       1       1	         0       0       0       0	   2 03	    0	   4 (4,64)

T3:    0     0       0       0       0       0           0       0       0       0	   3 00	    0	   5

-----------------------------------------------------------------------------------------------------------------------------------

T1:    1     0	     x       x       x       x           0       1       1       1	   1 2X	    7	   6

T2:    0     0	     0       1       1       1           0       0       1       1	   2 07	    3	   7

T3:    0     0	     0       0       1       1	         0       0       0       0	   3 03	    0	   8 (8,69)

T0:    0     x       0       0       0       0           0       0       0       0	   0 00     0	   9 (9,21)

:											   0 10     0      10 (10,22)

-----------------------------------------------------------------------------------------------------------------------------------

T2:    1     0	     x       x       x       x           0       1       1       1	   2 2X	    7	   11

T3:    0     0	     0       1       1       1           0       0       1       1	   3 07	    3	   12

T0:    0     x	     0       0       1       1	         0       0       0       0	   0 03	    0	   13

:											   0 13     0      14

T1:    0     0       0       0       0       0           0       0       0       0	   1 00	    0	   15

-----------------------------------------------------------------------------------------------------------------------------------

T3:    1     0	     x       x       x       x           0       1       1       1	   3 2X	    7	   16

T0:    0     x	     0       1       1       1           0       0       1       1	   0 07     3	   17

:											   0 17     3      18

T1:    0     0	     0       0       1       1	         0       0       0       0	   1 03	    0	   19

T2:    0     0       0       0       0       0           0       0       0       0	   2 00	    0	   20

-----------------------------------------------------------------------------------------------------------------------------------

-----------------------------------------------------------------------------------------------------------------------------------

If the VG wants the memory and the 68010 is not using it, give it to the VG.

VGSEL VFETCH VGSTALL VGDTACK  VBUFF    V68   >>  VGSTALL VGDTACK  VBUFF    V68    	   T Addr  Data   (State)

----- ------ ------- -------  -----   -----      ------- -------  -----   -----       ------  ----   -------

T0:    0     x	     0       0       0       0           0       0       0       0	   0 00     0	   21 (9,21)

0 10     0	   22 (10,22)

T1:    0     1	     0       0       0       0           0       0       0       0	   1 10     0	   23 (23,33,39,49,58)

T2:    0     1	     0       0       0       0	         0       0       0       0	   2 10	    0	   24

T3:    0     1       0       0       0       0           0       0       0       0	   3 10	    0	   25

---------------------------------------------------------------------------------------------------------------------------------

---------------------------------------------------------------------------------------------------------------------------------

If the 68010 wants the memory during 1.5 MHz low, and the VG wants it, give it to the 68010 and stall the VG.

VGSEL VFETCH VGSTALL VGDTACK  VBUFF    V68   >>  VGSTALL VGDTACK  VBUFF    V68    	   T Addr  Data   (State)

----- ------ ------- -------  -----   -----      ------- -------  -----   -----       ------  ----   -------

T0:    1     x	     0       0       0       0           0       1       1       1	   0 20     7	   26 (1,26)

0 30     7	   27 (2,27)

T1:    0     1	     0       1       1       1           1       0       1       1	   1 17	    B	   28 (28,44,53)

T2:    0     1	     1       0       1       1	         1       0       0       0	   2 1B	    8	   29 (29,45,54)

T3:    0     1       1       0       0       0           1       0       0       0	   3 18	    8	   30 (30,46,55)

T0:    0     x       1       0       0       0	         0       0       0       0	   0 08	    0      31 (31,37,47,56)

:											   0 18     0      32 (32,38,48,57)

T1:    0     1       0       0       0       0           0       0       0       0         1 10     0      33 (23,33,39,49,58)

-----------------------------------------------------------------------------------------------------------------------------------

T1:    1     1	     0       0       0       0           1       1       1       1	   1 30	    F	   34

T2:    0     1	     1       1       1       1           1       0       1       1	   2 1F	    B	   35

T3:    0     1	     1       0       1       1	         1       0       0       0	   3 1B	    8	   36

T0:    0     x       1       0       0       0           0       0       0       0         0 08     0	   37 (31,37,47,56)

:											   0 18     0      38 (32,38,48,57)

T1:    0     1       0       0       0       0           0       0       0       0	   1 10	    0	   39 (23,33,39,49,58)

-----------------------------------------------------------------------------------------------------------------------------------

If the 68010 wants the memory during 1.5 MHz high, and the VG is using it, make the 68010 wait until T0.

Next VG cycle is memory.  Stall the next VG cycle (even if it isn't strictly necessary).

VGSEL VFETCH VGSTALL VGDTACK  VBUFF    V68   >>  VGSTALL VGDTACK  VBUFF    V68    	   T Addr  Data   (State)

----- ------ ------- -------  -----   -----      ------- -------  -----   -----       ------  ----   -------

T2:    1     1	     x       x       x       x           0       0       0       1	   2 3X	    1	   40 (40,59)

T3:    0     1	     0       0       0       1           0       0       0       1	   3 11	    1	   41 (41,60)

T0:    0     x	     0       0       0       1	         0       1       1       1	   0 01     7	   42 (42,51,61,66)

:											   0 11     7 	   43 (43,52,62,67)

T1:    0     1       0       1       1       1           1       0       1       1	   1 17	    B	   44 (28,44,53)

T2:    0     1	     1       0       1       1	         1       0       0       0	   2 1B	    8	   45 (29,45,54)

T3:    0     1       1       0       0       0           1       0       0       0         3 18     8      46 (30,46,55)

T0:    0     x       1       0       0       0           0       0       0       0         0 08     0      47 (31,37,47,56)

:                                                                                        0 18     0      48 (32,38,48,57)

T1:    0     1       0       0       0       0           0       0       0       0         1 10     0      49 (23,33,39,49,58)

-----------------------------------------------------------------------------------------------------------------------------------

T3:    1     1	     x       x       x       x           0       0       0       1	   3 3X	    1	   50 (50,65)

T0:    0     x	     0       0       0       1           0       1       1       1	   0 01     7	   51 (42,51,61,66)

0 11     7	   52 (43,52,62,67)

T1:    0     1	     0       1       1       1	         1       0       1       1	   1 17	    B	   53 (28,44,53)

T2:    0     1       1       0       1       1           1       0       0       0	   2 1B	    8	   54 (29,45,54)

T3:    0     1       1       0       0       0           1       0       0       0         3 18     8      55 (30,46,55)

T0:    0     x       1       0       0       0           0       0       0       0         0 08     0      56 (31,37,47,56)

:                                                                                        0 18     0      57 (32,38,48,57)

T1:    0     1       0       0       0       0           0       0       0       0         1 10     0      58 (23,33,39,49,58)

-----------------------------------------------------------------------------------------------------------------------------------

-----------------------------------------------------------------------------------------------------------------------------------

If the 68010 wants the memory during 1.5 MHz high, and the VG is using it, make the 68010 wait until T0.

Next VG cycle is not memory.

VGSEL VFETCH VGSTALL VGDTACK  VBUFF    V68   >>  VGSTALL VGDTACK  VBUFF    V68    	   T Addr  Data   (State)

----- ------ ------- -------  -----   -----      ------- -------  -----   -----       ------  ----   -------

T2:    1     1	     x       x       x       x           0       0       0       1	   2 3X	    1	   59 (40,59)

T3:    0     1	     0       0       0       1           0       0       0       1	   3 11	    1	   60 (41,60)

T0:    0     x	     0       0       0       1	         0       1       1       1	   0 01     7	   61 (42,51,61,66)

0 11     7 	   62 (43,52,62,67)

T1:    0     0       0       1       1       1           0       0       1       1	   1 07	    3	   63 (3,63,68)

T2:    0     0	     0       0       1       1	         0       0       0       0	   2 03	    0	   64 (4,64)

-----------------------------------------------------------------------------------------------------------------------------------

T3:    1     1	     x       x       x       x           0       0       0       1	   3 3X	    1	   65 (50,65)

T0:    0     x	     0       0       0       1           0       1       1       1	   0 01     7	   66 (42,51,61,66)

0 11     7	   67 (43,52,62,67)

T1:    0     0	     0       1       1       1	         0       0       1       1	   1 07	    3	   68 (3,63,68)

T2:    0     0       0       0       1       1           0       0       0       0	   3 03	    0	   69 (8,69)

-----------------------------------------------------------------------------------------------------------------------------------

-----------------------------------------------------------------------------------------------------------------------------------

Everything else:					 0       0       0       0