To: Dan van Eldren

Fr: Jed Margolin

Dt: 3-4-86

Re: VG Gate Array

--------------------------------------------------------------------------------

Information from Erwin Arvidson:

He has been talking to AMI's Bob Rush whom he has quoted as saying

that he (Rush) vaguely remembers talking to someone from Atari about it

but he couldn't believe that AMI had actually scrapped everything. He

was supposed to get back to Erwin but hasn't yet.

Either Bob Rush is lying to Erwin or he lied to me when I talked

to him in December.

--------------------------------------------------------------------------------

--------------------------------------------------------------------------------

Information from Don Paauw:

The new VTI package is in 2u CMOS compared to the previous package

which was 3u NMOS.

This new CMOS is somewhat smaller than the old NMOS and runs about

twice the speed (20 MHz for CMOS versus 10 MHz for the NMOS).

The new CMOS package has a bunch of high level standard cells. The old

NMOS does not (and has lots of bugs which only the "vetrans" know about).

The engineering charge from VTI for making the mask set and making

engineering samples is $28,000.

Doug Snyder's company will do it for $7,000 but can only do the older

3u NMOS.

For a 40 pin IC with a relatively small die, the price from VTI

for packaged and tested ICs would run about $4 .

--------------------------------------------------------------------------------

An oops! board with (3) 74LS670, (3) 74LS193, (1) 74LS191, (2) 74LS367,

(1) 74LS32, (1) 74S04, (1) 74LS74 would cost approximately $15 .

It would take about six months of Atari time to design the IC. Assign

salary and overhead of $62,000 .

Compare costs:

oops! board = n x $15 + $1000 (PCB designer and overhead for 1 week.

VTI	    = n x $4  + $28,000 (VTI Engineering)

+   $62,000 (Atari Engineering)

The breakeven point is when these two costs are equal.

15 x N + 1000  = 4 x N + 28,000 + 62,000

15 x N - 4 x N = 28,000 + 62,000 - 1000

11 x N = 80,000

N = 80,000 / 11 = 7,273

For fewer than 7,273 units it is cheaper to use the oops! board. There

are, however three additional factors:

1. An oops! board is unesthetic.

2. The time spent designing the IC could probably be

more profitably spent on other things.

3. This problem exists only because AMI scrapped all of the tooling

(and data) for the Gate Array we bought from them.

--------------------------------------------------------------------------------

Solution: Sue the bastards.