TC68.DOC

Started 3/85

Jed Margolin

Timing Documentation (6 MHZ 68010, 16 MHz TMS320):

-------------------------------------------------------------------------------

To Set up, Call, and Return from:

Inituv (Initialise Unit Vectors): 94 uS.

Concatenation: 			330 uS. 68010 only

180 uS. with TMS320

[ 680 uS. for 16032 ]

[ 100 uS raw SW ]

Post Add 1 (and project):	4.5 mS/25 points = 180 uS/point

68010 only

(40 uS is just for raw divides.)

72 uS/point TMS320 semiautomatic

50 uS/point TMS320 automatic

[ 336 uS/point for 16032 ]

[ 70 uS/point raw SW]

Generate the Working Copy

(Copy, Roll, Pitch, Yaw)	620 uS.

Format Rocket:			640 uS./~60 lines  = ~10.6 uS/line

[ 33 uS/line for 16032 ]

[ 50 uS.line SW 6809]

For a grid of 22 lines:

All visible	Ground View	None Visible

(no clipping)

-------------	------------	-------------

All 68010	   10.0 ms	    14.0 ms

320 Level 1	    6.0 ms	     9.0 ms

320 Level 2	    5.4 ms	     6.2 ms	     3.5 ms

320 Level 3	    4.6 ms	     5.5 ms	     2.8 ms

(The TMS-320 routines were brought up in three stages. Level 3

uses more of the TMS-320 routines than Levels 1 or 2.)

Vector Drawing Time for the box to fool the beam killer: 1.2 mS

Vector Drawing Time for the half-screen Rocket Ship:	 1.4 mS

For a naked 68010 to calculate the rocket takes (per update) at least:

Concatenation: 	0.33 mS.

Post Add #1:	4.5 mS.

Format Rocket:	0.64 mS.

1 PreSubtract:	0.18 mS.(est)

________

5.65 mS.

This is about 4 times as long as it takes to draw it.

(for a rocket ship that is half the size of the screen.

ROLL or PITCH or YAW =  170 uS	68010 only

140 uS TMS320 Level 1

[ 71 uS raw SW ]

Vector Generator Memory:  Terrain Objects, Horizon Line, Headup Display,

Rocket Ship, Grid (11 * 11) = 1150. Bytes

Terrain Objects (all 68010) :

1. Calculation time for Terrain Objects:

[Rotate (POST2), Object Center Point (PRESUB),and Projection (TPROJ)]

10 Cubes: 10.2 mS / 10 Objects / 8 points/object = 127.5 uS/point

(40 uS is just for two raw divides.)

10 Tanks: 27 mS / 10 objects /24 points/object = 112.5 uS/point

2. TPROJ  Terrain Translation and Projection:

Cube:	560 uS/8 points = 70 uS/point	      [ 105 uS/point for 16032 ]

Tank:   1.6 mS/24 points = 66 uS/point

3. Vector Drawing Times:

Box to fool the Beam Killer: 	1.2 mS	(30 bytes of VG Memory)

Half-screen Rocket Ship:	1.4 mS	(212 bytes of VG Memory)

10 Terrain Cubes: 		1.2 mS	(700 bytes of VG Memory)

10 Terrain Tanks:		5.8 mS	(1898 bytes of VG Memory)

4. POST2 Rotate Terrain Object		920 uS/8 points	= 115 uS/point

[ 270 uS. for 16032 ]

5. PRESUB Presubtract (center point)			= 180 uS/point

[ 260 uS. for 16032 ]

So far, it takes longer to calculate the points than it does to draw them.

It probably depends somewhat on the type of terrain object.

-------------------------------------------------------------------------------

Calculate Horizon: 	260 uS		[ 550 uS. for 16032 ]	[ 66 uS raw SW]

-------------------------------------------------------------------------------

Head Up Display:  1.0 mS, includes Heading, Airspeed, Altitude

[ 2.0 mS. for 16032 ]

Heading:	200 uS  Calculate heading in degrees and convert to BCD.

[ 480 uS. for 16032 ]

140 uS  Format as a moving horizontal tape.

[ 270 uS. for 16032 ]

Altitude:	___ uS	Display as a moving vertical tape. [ 680 uS. for 16032 ]

Airspeed:	___ uS	Calculate Indicated (Pitot tube) Airspeed,

Display as a moving vertical tape. [ 700 uS. for 16032 ]

TTL 'CLIP Against 5 Clipping Planes'

INCLUDE 'COMN.ASM'

SECTION PROGRAM

XDEF GRID,GRLINE,XMIN

XREF M_GRLINE

X1	EQU	PTBL+$0

Y1	EQU	PTBL+$2

Z1	EQU	PTBL+$4

X2	EQU	PTBL+$6

Y2	EQU	PTBL+$8

Z2	EQU	PTBL+$0A

T	EQU	PTBL+$0C

HGL	MACRO			* Horizontal Grid Line

DC.W	\1*GRK,-GRSIZE*GRK,0

DC.W	\1*GRK,GRSIZE*GRK,0

ENDM

VGL	MACRO			* Vertical Grid Line

DC.W	-GRSIZE*GRK,\1*GRK,0

DC.W	 GRSIZE*GRK,\1*GRK,0

ENDM

GRK	EQU	$800		* Grid Scale Constant

GRSIZE	EQU	$5		* Grid Size

GRLINE:

HGL	-$5

HGL	-$4

HGL	-$3

HGL	-$2

HGL	-$1

HGL	0

HGL	$1

HGL	$2

HGL	$3

HGL	$4

HGL	$5

VGL	-$5

VGL	-$4

VGL	-$3

VGL	-$2

VGL	-$1

VGL	0

VGL	$1

VGL	$2

VGL	$3

VGL	$4

VGL	$5

GPTS	EQU	22*2		* Grid Points = Grid Lines * 2

XMIN	DC.W	$100