Observations on 68010 timing:

A lot of the problems could be avoided by using an L8 at 6 MHz, but

then everything has to be redone when I speed it up.

-------------------------------------------------------------------------------

Address valid to /AS low could be as short as 30 ns (L8), 20 nS (L10),

or 0 ns (L12).

/AS high to address invalid could be as short as 30 ns (L8), 20 ns (L10),

or 10 ns (L12).

As they refine their process they will probably all end up as L12s.

(But will they sell L12s marked as L8s?)

Who knows what is in the development system?

In any event, I might want to eventually run at 12 MHz, anyway.

There is a good chance that after address buffering and decoding /AS will end

up low before the address is really valid or will not have enough time to

properly select a decoder (LS138) without glitches. (Some Binary Selects go

through three levels of logic (tPHL = 39 ns) while some Enables go through two

(tPHL = 32 ns). Therefore, if a three level Binary Select changes at the same

time as an Enable, the decoder will probably glitch.

During Reads the Data Hold time is 0 (from /AS [/DS] high).

As long as Reads are non-destructive, /AS can just be ignored.

The minimum /AS width for a Read is 240 ns (L8 at 8 MHz). But since

the Address is guaranteed valid 70 ns after S0 and Data Setup is 15 ns

before the end of S6, the real memory access time is 62.5*6 - 70 - 15 = 290 ns.

(again, at 8 MHz.). For non-destructive Reads such as ROMs, /AS should

certainly be ignored.

Also, the Address is guaranteed valid 70 ns into S1 (L8), and 55 ns (L10 and

L12). At 6 MHz this leaves 83.3 - 70 = 13.3 ns left from S1 (L8). But an L8

at 8 MHz may not have a valid address until 62.5 - 70 = 7.5 ns into S2. But

it could instead be valid before S2 because the minimum is not specified.

(Presumably, it is 0 because it is shown as referenced to the beginning of S1.)

An L12 running at 12 MHz might not have a valid address until 41.7 - 55 =

13.3 ns into S2. This could be suspicious. Perhaps address valid is actually

clocked by S2.

-------------------------------------------------------------------------------

Writes to RAM:

Reads are non-destructive, so just decode the address. /AS is not required.

Because the minimum /AS (/DS) to Address invalid is 30 ns (L8), 20 ns (L10),

and 10 ns (L12), the address decoders only have that much time to turn off

(plus address and data buffer propagation delay.)

Writes are destructive if the address is not properly stable. /AS cannot be

used, because it might be asserted before the address decoders are stable and

it might not have time to turn the address decoders off before the address and

data start to go away. Therefore do not connect R/W to RAM /WE and hope

/CS gets there before address and data go away.

/UDS and /LDS can be ANDed with R/W using Shottky gates to produce valid

Write strobes for the RAMs. (Maybe ALS gates).

[I ended up generating my own Really Valid Write Strobe (RWS) which begins

with RVA and ends with the falling edge of S6 (no waiting) or on the falling

edge following the edge that recognizes /DTACK. This allows normal address

decoders to be used with write strobes.]

-------------------------------------------------------------------------------

Writes to TTL:

Generate a signal that is low during S4 and S5. This will be called

/RVA (Real Valid Address). It will generate a strobe (166 ns wide

at 6 Mhz and 83 ns wide at 12 MHz). Data will be valid for at least the

last half of the strobe and will be clocked in on the rising edge of the

signal. If necessary the strobe can be widened by one state with more

complicated circuitry.

Make sure no wait states are inserted when using /RVA.

-------------------------------------------------------------------------------

For Reads from TTL:

Just decode the address.

For Reads that reset a flag (the parallel interface with full handshaking):

Address decodes from /AS won't be reliable. Use /RVA to reset the

flag. The actual read will be done normally.

-------------------------------------------------------------------------------

Vector Generator Memory:

1. Quantum inserts wait states until the beginning of 1.5 MHz, then does a

traditional VG Stall. It does it even when the VG is not doing a memory

access and this method therefore wastes time.

2. VGSTAT.LDA contains the ROM code for a State PROM memory arbitrator that

minimizes the amount of time the 68010 spends waiting for the VG memory.

3. Use two separate 16K hardware buffers. The 68010 will be writing into one

while the VG is executing from the other. This has the advantage of

increasing the amount of VG memory and also permits the 68010 and the VG to

operate at different clock frequencies.

-------------------------------------------------------------------------------

Address Decoders and Data Buffers:

LS04   15	ALS04	11	S04	5

LS138  41	ALS138  18	S138   12 But address buffer is 18

LS244  18	LS244   18	LS244  18

---	       ---	       ---

74	        47	       41

tacc    Memory    Waits  TACC

----    ------    -----------

6 MHz:  83.3*6 - 70 - 15 = 414,  414 - 74 = 340	     300	0    340 <<<<<<<

8 MHz:  62.5*6 - 70 - 15 = 290,  290 - 74 = 216	     200?	0    216

290 - 47 = 243	     200	0    243 <<<<<<<

10 MHz: 50.0*6 - 55 - 15 = 230,  230 - 74 = 156	     150?	0    156

200?	1    206

230 - 47 = 183	     150	0    183 <<<<<

200	1    233 <<<<<

12 MHz: 41.7*6 - 55 - 15 = 180,  180 - 74 = 106	     200	3    231

180 - 47 = 133	     200	2    216

150	1    175

180 - 41 = 139	     200	2    223

150	1    181

27128 EPROM:	137328-001	200 ns

137328-004	250 ns

137328-002	300 ns

137328-003	450 ns

For RVA Generator:   Prefer AS04, AS74

6 MHz,  use LS04, LS74; or ALS04, ALS74; or S04, S74; or AS04, AS74

8 MHz,  use S04, LS74; or LS04, ALS74; or LS04, S74; or AS04, AS74

10 MHz: use S04, ALS74; or S04, S74; or AS04, AS74

12 MHz: use S04, S74; or AS04, AS74

74ALS04	  137157-001

74ALS74	  137156-001

68010 prices 3/11/85:		10 MHz (Plastic)      12.5 MHz (Ceramic)

----------------      ------------------

5K		   $29.50		  $70.50

20K		   $25.

68008 maybe $12.

They do not expect the 12.5 MHz part to be in plastic because of the

heat dissipation.

We are currently using 68010-L8	in quantities of 20K for $20.